#include <SPI.h>

#include <nRF24L01.h>

#include <RF24.h>

// Third Arduino Nano: USB radio bridge for the laptop dashboard.

// Connect an nRF24L01 to this Nano, upload this sketch, then open the

// dashboard page in Chrome/Edge and connect to this Nano over Web Serial.

#define CE_PIN 9

#define CSN_PIN 10

#define PELTIER_OFF 0

#define PELTIER_COLD 1

#define PELTIER_HEAT 2

#define LIGHTS_OFF 0

#define LIGHTS_BOTH_BLINK 1

#define LIGHTS_RIGHT_BLINK 2

#define LIGHTS_LEFT_BLINK 3

#define LIGHTS_BOTH_ON 4

RF24 radio(CE_PIN, CSN_PIN);

const uint64_t RADIO_ADDRESS = 0xE9E8F0F0E1LL;

const uint64_t DASHBOARD_ADDRESS = 0xE9E8F0F0D2LL;

const uint64_t DASHBOARD_CONFIG_ADDRESS = 0xE9E8F0F0C3LL;

const byte RADIO_CHANNEL = 76;

const uint8_t DASHBOARD_CONFIG_MAGIC = 0xC3;

struct __attribute__((packed)) JoystickPacket {

uint16_t x;

uint16_t y;

uint8_t peltierMode;

uint8_t buzzerActive;

uint8_t fanActive;

uint8_t lightsActive;

};

struct __attribute__((packed)) DashboardConfigPacket {

uint8_t magic;

uint16_t buzzerToneHz;

};

struct __attribute__((packed)) TelemetryPacket {

uint16_t x;

uint16_t y;

uint8_t peltierMode;

uint8_t buzzerActive;

uint8_t fanActive;

uint8_t lightsActive;

uint16_t servoPulse;

uint8_t motorPWM;

int16_t temperatureC10;

int16_t humidity10;

uint8_t sensorValid;

uint8_t failsafe;

uint16_t signalAgeMs;

uint32_t packetCount;

uint16_t buzzerToneHz;

};

char serialBuffer[40];

byte serialBufferIndex = 0;

const char *peltierModeName(uint8_t mode) {

switch (mode) {

case PELTIER_COLD:

return "COLD";

case PELTIER_HEAT:

return "HEAT";

default:

return "OFF";

}

}

uint8_t safeLightMode(uint8_t mode) {

if (mode > LIGHTS_BOTH_ON) {

return LIGHTS_OFF;

}

return mode;

}

const char *lightModeName(uint8_t mode) {

switch (safeLightMode(mode)) {

case LIGHTS_BOTH_BLINK:

return "BOTH_BLINK";

case LIGHTS_RIGHT_BLINK:

return "RIGHT_BLINK";

case LIGHTS_LEFT_BLINK:

return "LEFT_BLINK";

case LIGHTS_BOTH_ON:

return "BOTH_ON";

default:

return "OFF";

}

}

void restoreDashboardListening() {

radio.setAutoAck(false);

radio.openReadingPipe(1, RADIO_ADDRESS);

radio.openReadingPipe(2, DASHBOARD_ADDRESS);

radio.startListening();

}

void printBoolJson(const char *key, uint8_t value, bool trailingComma) {

Serial.print("\"");

Serial.print(key);

Serial.print("\":");

Serial.print(value ? "true" : "false");

if (trailingComma) {

Serial.print(",");

}

}

void printLightModeJson(uint8_t mode, bool trailingComma) {

uint8_t safeMode = safeLightMode(mode);

Serial.print("\"lightsActive\":");

Serial.print(safeMode);

Serial.print(",\"lightsMode\":\"");

Serial.print(lightModeName(safeMode));

Serial.print("\"");

if (trailingComma) {

Serial.print(",");

}

}

void printTenthsJson(const char *key, int16_t value, bool valid, bool trailingComma) {

Serial.print("\"");

Serial.print(key);

Serial.print("\":");

if (!valid || value == -32768) {

Serial.print("null");

} else {

Serial.print(value / 10.0, 1);

}

if (trailingComma) {

Serial.print(",");

}

}

void printCommandJson(const JoystickPacket &packet) {

Serial.print("{\"type\":\"command\",\"ms\":");

Serial.print(millis());

Serial.print(",\"x\":");

Serial.print(packet.x);

Serial.print(",\"y\":");

Serial.print(packet.y);

Serial.print(",\"peltierMode\":\"");

Serial.print(peltierModeName(packet.peltierMode));

Serial.print("\",");

printBoolJson("buzzerActive", packet.buzzerActive, true);

printBoolJson("fanActive", packet.fanActive, true);

printLightModeJson(packet.lightsActive, false);

Serial.println("}");

}

void printTelemetryJson(const TelemetryPacket &packet) {

Serial.print("{\"type\":\"telemetry\",\"ms\":");

Serial.print(millis());

Serial.print(",\"x\":");

Serial.print(packet.x);

Serial.print(",\"y\":");

Serial.print(packet.y);

Serial.print(",\"peltierMode\":\"");

Serial.print(peltierModeName(packet.peltierMode));

Serial.print("\",");

printBoolJson("buzzerActive", packet.buzzerActive, true);

printBoolJson("fanActive", packet.fanActive, true);

printLightModeJson(packet.lightsActive, true);

Serial.print("\"servoPulse\":");

Serial.print(packet.servoPulse);

Serial.print(",\"motorPWM\":");

Serial.print(packet.motorPWM);

Serial.print(",");

printTenthsJson("temperatureC", packet.temperatureC10, packet.sensorValid, true);

printTenthsJson("humidity", packet.humidity10, packet.sensorValid, true);

printBoolJson("sensorValid", packet.sensorValid, true);

printBoolJson("failsafe", packet.failsafe, true);

Serial.print("\"signalAgeMs\":");

Serial.print(packet.signalAgeMs);

Serial.print(",\"packetCount\":");

Serial.print(packet.packetCount);

Serial.print(",\"buzzerToneHz\":");

Serial.print(packet.buzzerToneHz);

Serial.println("}");

}

void printConfigJson(uint16_t toneHz, bool ok) {

Serial.print("{\"type\":\"config\",\"ms\":");

Serial.print(millis());

Serial.print(",\"buzzerToneHz\":");

Serial.print(toneHz);

Serial.print(",\"ok\":");

Serial.print(ok ? "true" : "false");

Serial.println("}");

}

void sendBuzzerConfig(uint16_t toneHz) {

toneHz = constrain(toneHz, 200, 5000);

DashboardConfigPacket configPacket;

configPacket.magic = DASHBOARD_CONFIG_MAGIC;

configPacket.buzzerToneHz = toneHz;

radio.stopListening();

radio.setAutoAck(true);

radio.openWritingPipe(DASHBOARD_CONFIG_ADDRESS);

bool ok = radio.write(&configPacket, sizeof(configPacket));

restoreDashboardListening();

printConfigJson(toneHz, ok);

}

void processSerialCommand(char *line) {

while (*line == ' ' || *line == '\t') {

line++;

}

if (strncmp(line, "BUZZER", 6) == 0 || strncmp(line, "buzzer", 6) == 0) {

char *numberStart = line + 6;

while (*numberStart && (*numberStart < '0' || *numberStart > '9')) {

numberStart++;

}

int requestedTone = atoi(numberStart);

if (requestedTone > 0) {

sendBuzzerConfig((uint16_t)requestedTone);

}

} else if (strncmp(line, "PING", 4) == 0 || strncmp(line, "ping", 4) == 0) {

Serial.println("{\"type\":\"bridge\",\"status\":\"ok\"}");

}

}

void readSerialCommands() {

while (Serial.available()) {

char c = Serial.read();

if (c == '\r') {

continue;

}

if (c == '\n') {

serialBuffer[serialBufferIndex] = '\0';

processSerialCommand(serialBuffer);

serialBufferIndex = 0;

continue;

}

if (serialBufferIndex < sizeof(serialBuffer) - 1) {

serialBuffer[serialBufferIndex++] = c;

}

}

}

void setup() {

Serial.begin(115200);

delay(500);

Serial.println("{\"type\":\"bridge\",\"status\":\"starting\"}");

if (!radio.begin()) {

Serial.println("{\"type\":\"bridge\",\"status\":\"radio_error\"}");

while (1) {}

}

radio.setChannel(RADIO_CHANNEL);

radio.setDataRate(RF24_250KBPS);

radio.setPALevel(RF24_PA_LOW);

restoreDashboardListening();

Serial.println("{\"type\":\"bridge\",\"status\":\"ready\"}");

}

void loop() {

readSerialCommands();

uint8_t pipeNum = 0;

while (radio.available(&pipeNum)) {

if (pipeNum == 1) {

JoystickPacket packet;

radio.read(&packet, sizeof(packet));

printCommandJson(packet);

} else if (pipeNum == 2) {

TelemetryPacket packet;

radio.read(&packet, sizeof(packet));

printTelemetryJson(packet);

} else {

radio.flush_rx();

}

}

}

<!doctype html>

<html lang="en">

<head>

<meta charset="utf-8">

<meta name="viewport" content="width=device-width, initial-scale=1">

<title>VANT151 TEAM1 2026 RC Dashboard</title>

<style>

:root {

color-scheme: light;

--bg: #eceeef;

--surface: #ffffff;

--surface-soft: #f6f6f6;

--ink: #24272c;

--muted: #676b73;

--line: #d6d8dc;

--line-strong: #b6bbc2;

--accent: #a51c30;

--accent-hover: #8f1729;

--accent-soft: #f5e6e9;

--accent-softer: #fbf2f3;

--graph-gray: #747982;

--warn: #a51c30;

--warn-soft: #f8e8eb;

--danger: #7c1020;

--danger-soft: #f4dce1;

--ok: #424850;

--ok-soft: #eef0f2;

--mono: "JetBrains Mono", "SFMono-Regular", Consolas, monospace;

--sans: "Geist", "Satoshi", "Segoe UI", Arial, sans-serif;

}

* {

box-sizing: border-box;

}

body {

margin: 0;

min-width: 320px;

background:

radial-gradient(circle at 14% 0%, rgba(165, 28, 48, 0.08), transparent 28%),

linear-gradient(180deg, #f4f4f4 0%, var(--bg) 48%, #e3e5e8 100%);

color: var(--ink);

font-family: var(--sans);

letter-spacing: 0;

}

button,

input {

font: inherit;

}

button {

display: inline-flex;

align-items: center;

justify-content: center;

min-height: 38px;

border: 1px solid var(--accent);

border-radius: 6px;

background: var(--accent);

color: #ffffff;

padding: 0 14px;

font-size: 14px;

font-weight: 750;

cursor: pointer;

transition: transform 180ms ease, background 180ms ease, border-color 180ms ease, opacity 180ms ease;

}

button:hover:not(:disabled) {

background: var(--accent-hover);

border-color: var(--accent-hover);

}

button:active:not(:disabled) {

transform: translateY(1px);

}

button.secondary {

background: #ffffff;

color: var(--accent);

}

button.secondary:hover:not(:disabled) {

background: var(--accent-soft);

color: var(--accent);

}

button:disabled {

cursor: not-allowed;

opacity: 0.52;

}

.page {

width: min(1280px, calc(100vw - 32px));

margin: 0 auto;

padding: 22px 0 34px;

}

.app-header {

display: grid;

grid-template-columns: minmax(0, 1fr) auto;

gap: 18px;

align-items: end;

padding: 22px 0 16px;

border-top: 5px solid var(--accent);

border-bottom: 1px solid var(--line-strong);

}

.eyebrow {

margin: 0 0 7px;

color: var(--accent);

font-size: 12px;

font-weight: 800;

letter-spacing: 0.08em;

text-transform: uppercase;

}

h1 {

margin: 0;

font-size: clamp(28px, 4vw, 44px);

line-height: 1.03;

font-weight: 850;

letter-spacing: 0;

}

.lead {

max-width: 760px;

margin: 10px 0 0;

color: var(--muted);

font-size: 15px;

line-height: 1.5;

}

.toolbar {

display: flex;

align-items: center;

justify-content: flex-end;

gap: 10px;

flex-wrap: wrap;

}

.connection-pill {

display: inline-grid;

grid-template-columns: 9px auto;

gap: 9px;

align-items: center;

min-height: 38px;

border: 1px solid var(--line);

border-radius: 999px;

background: rgba(255, 255, 255, 0.82);

padding: 0 13px;

color: var(--muted);

font-size: 13px;

font-weight: 750;

white-space: nowrap;

}

.dot {

width: 9px;

height: 9px;

border-radius: 999px;

background: var(--line-strong);

}

.dot.online {

background: var(--accent);

animation: breathe 1.8s ease-in-out infinite;

}

.dot.warn {

background: var(--warn);

}

.dot.danger {

background: var(--danger);

}

@keyframes breathe {

0%, 100% {

opacity: 0.55;

transform: scale(0.92);

}

50% {

opacity: 1;

transform: scale(1);

}

}

.signal-strip {

display: grid;

grid-template-columns: repeat(4, minmax(0, 1fr));

gap: 10px;

margin: 16px 0;

}

.node {

display: grid;

grid-template-columns: 32px minmax(0, 1fr);

gap: 10px;

align-items: center;

min-height: 64px;

border: 1px solid var(--line);

border-radius: 8px;

background: rgba(255, 255, 255, 0.84);

padding: 10px;

}

.node-number {

display: inline-flex;

align-items: center;

justify-content: center;

width: 32px;

height: 32px;

border-radius: 6px;

background: var(--accent);

color: #ffffff;

font-family: var(--mono);

font-size: 13px;

font-weight: 800;

}

.node-title {

color: var(--ink);

font-size: 13px;

font-weight: 800;

}

.node-sub {

margin-top: 3px;

color: var(--muted);

font-size: 12px;

line-height: 1.35;

}

.dashboard-layout {

display: grid;

grid-template-columns: minmax(0, 1.65fr) minmax(320px, 0.85fr);

gap: 14px;

align-items: start;

}

.main-grid {

display: grid;

grid-template-columns: repeat(4, minmax(0, 1fr));

gap: 12px;

}

.panel {

border: 1px solid var(--line);

border-radius: 8px;

background: rgba(255, 255, 255, 0.92);

box-shadow: 0 18px 44px -34px rgba(36, 39, 44, 0.42);

}

.metric {

min-height: 138px;

padding: 14px;

}

.metric.wide {

grid-column: span 2;

}

.metric.tall {

grid-row: span 2;

}

.panel-header {

display: flex;

align-items: center;

justify-content: space-between;

gap: 12px;

margin-bottom: 12px;

}

h2 {

margin: 0;

color: var(--muted);

font-size: 12px;

font-weight: 850;

letter-spacing: 0.07em;

text-transform: uppercase;

}

.value {

display: flex;

align-items: baseline;

gap: 5px;

min-height: 42px;

font-family: var(--mono);

font-size: clamp(28px, 4vw, 38px);

font-weight: 850;

line-height: 1.05;

white-space: nowrap;

}

.unit {

color: var(--muted);

font-family: var(--sans);

font-size: 14px;

font-weight: 800;

}

.sub {

margin-top: 10px;

color: var(--muted);

font-size: 13px;

line-height: 1.35;

min-height: 18px;

}

.chip {

display: inline-flex;

align-items: center;

justify-content: center;

min-width: 82px;

min-height: 30px;

border: 1px solid var(--line);

border-radius: 999px;

background: #eff0f2;

color: #343941;

padding: 0 11px;

font-size: 13px;

font-weight: 850;

}

.chip.ok {

border-color: #c9cdd3;

background: var(--ok-soft);

color: var(--ok);

}

.chip.warn {

border-color: #e4a5af;

background: var(--warn-soft);

color: var(--warn);

}

.chip.danger {

border-color: #d98d9a;

background: var(--danger-soft);

color: var(--danger);

}

.meter {

width: 100%;

height: 9px;

margin-top: 16px;

overflow: hidden;

border-radius: 999px;

background: #e3e5e8;

}

.meter > span {

display: block;

width: 0%;

height: 100%;

border-radius: inherit;

background: var(--accent);

transition: width 220ms ease;

}

.meter.neutral > span {

background: var(--graph-gray);

}

.meter.warn > span {

background: var(--warn);

}

.status-grid {

display: grid;

grid-template-columns: repeat(2, minmax(0, 1fr));

gap: 10px;

margin-top: 14px;

}

.status-line {

border-top: 1px solid var(--line);

padding-top: 10px;

}

.status-label {

color: var(--muted);

font-size: 11px;

font-weight: 800;

letter-spacing: 0.06em;

text-transform: uppercase;

}

.status-value {

margin-top: 4px;

font-family: var(--mono);

font-size: 15px;

font-weight: 800;

}

.side-stack {

display: grid;

gap: 12px;

}

.side-panel {

padding: 14px;

}

.control-row {

display: grid;

grid-template-columns: minmax(0, 1fr) 92px auto;

gap: 10px;

align-items: end;

margin-top: 14px;

}

label {

display: block;

margin-bottom: 7px;

color: var(--muted);

font-size: 12px;

font-weight: 850;

letter-spacing: 0.06em;

text-transform: uppercase;

}

input[type="range"] {

width: 100%;

accent-color: var(--accent);

}

input[type="number"] {

width: 92px;

min-height: 38px;

border: 1px solid var(--line);

border-radius: 6px;

background: #ffffff;

color: var(--ink);

padding: 0 8px;

font-family: var(--mono);

font-weight: 800;

}

.stack-list {

display: grid;

gap: 9px;

margin-top: 12px;

}

.stack-row {

display: grid;

grid-template-columns: 96px minmax(0, 1fr);

gap: 12px;

border-top: 1px solid var(--line);

padding-top: 9px;

font-size: 13px;

line-height: 1.35;

}

.stack-key {

color: var(--muted);

font-weight: 850;

}

.stack-value {

color: var(--ink);

}

.log-panel {

grid-column: 1 / -1;

min-height: 230px;

padding: 14px;

}

pre {

min-height: 178px;

max-height: 260px;

margin: 0;

overflow: auto;

border: 1px solid var(--line);

border-radius: 6px;

background: #f7f7f7;

color: #252930;

padding: 10px;

font-family: var(--mono);

font-size: 12px;

line-height: 1.45;

white-space: pre-wrap;

}

.empty-log {

color: var(--muted);

}

@media (max-width: 1040px) {

.dashboard-layout {

grid-template-columns: 1fr;

}

.side-stack {

grid-template-columns: repeat(2, minmax(0, 1fr));

}

}

@media (max-width: 820px) {

.page {

width: min(100vw - 24px, 1280px);

}

.app-header {

grid-template-columns: 1fr;

}

.toolbar {

justify-content: flex-start;

}

.signal-strip,

.main-grid,

.side-stack {

grid-template-columns: 1fr;

}

.metric.wide {

grid-column: span 1;

}

.control-row {

grid-template-columns: 1fr;

}

input[type="number"] {

width: 100%;

}

}

</style>

</head>

<body>

<div class="page">

<header class="app-header">

<div>

<p class="eyebrow">TEAM 1 C-Subteam 2026</p>

<h1>RC Climate & Control Dashboard</h1>

<p class="lead">Local laptop dashboard for the third Arduino Nano radio bridge. Telemetry arrives as JSON over USB Serial from the nRF24L01 bridge.</p>

</div>

<div class="toolbar">

<button id="connectButton">Connect USB Bridge</button>

<button id="disconnectButton" class="secondary" disabled>Disconnect</button>

<div class="connection-pill"><span id="connectionDot" class="dot"></span><span id="connectionStatus">Not connected</span></div>

</div>

</header>

<section class="signal-strip" aria-label="System signal path">

<div class="node">

<span class="node-number">01</span>

<div>

<div class="node-title">Transmitter Nano</div>

<div class="node-sub">Joystick, buttons, nRF24 command packet</div>

</div>

</div>

<div class="node">

<span class="node-number">02</span>

<div>

<div class="node-title">Vehicle Receiver</div>

<div class="node-sub">Outputs, DHT sensor, telemetry packet</div>

</div>

</div>

<div class="node">

<span class="node-number">03</span>

<div>

<div class="node-title">USB Radio Bridge</div>

<div class="node-sub">Third Nano prints JSON lines</div>

</div>

</div>

<div class="node">

<span class="node-number">04</span>

<div>

<div class="node-title">Browser Dashboard</div>

<div class="node-sub">Web Serial API, live local display</div>

</div>

</div>

</section>

<main class="dashboard-layout">

<section class="main-grid" aria-label="Live telemetry">

<article class="panel metric">

<div class="panel-header">

<h2>Cabin Temperature</h2>

</div>

<div class="value"><span id="temperatureValue">--</span><span class="unit">&deg;C</span></div>

<div id="temperatureSub" class="sub">Waiting for DHT data</div>

</article>

<article class="panel metric">

<div class="panel-header">

<h2>Humidity</h2>

</div>

<div class="value"><span id="humidityValue">--</span><span class="unit">%</span></div>

<div id="humiditySub" class="sub">Waiting for DHT data</div>

</article>

<article class="panel metric">

<div class="panel-header">

<h2>Peltier Mode</h2>

</div>

<div id="peltierMode" class="chip">OFF</div>

<div id="peltierSub" class="sub">Mode from receiver telemetry</div>

</article>

<article class="panel metric">

<div class="panel-header">

<h2>Failsafe</h2>

</div>

<div id="failsafeChip" class="chip ok">CLEAR</div>

<div id="signalSub" class="sub">No timeout data yet</div>

<div class="meter warn"><span id="signalBar"></span></div>

</article>

<article class="panel metric wide">

<div class="panel-header">

<h2>Motion Control</h2>

<span id="packetMini" class="chip">PACKETS --</span>

</div>

<div class="status-grid">

<div class="status-line">

<div class="status-label">Motor PWM</div>

<div class="status-value"><span id="motorValue">--</span> / 255</div>

<div class="meter"><span id="motorBar"></span></div>

<div id="motorSub" class="sub">Joystick Y mapped by receiver</div>

</div>

<div class="status-line">

<div class="status-label">Steering Pulse</div>

<div class="status-value"><span id="servoValue">--</span> us</div>

<div class="meter neutral"><span id="steeringBar"></span></div>

<div id="joystickSub" class="sub">X: -- - Y: --</div>

</div>

</div>

</article>

<article class="panel metric">

<div class="panel-header">

<h2>Fan</h2>

</div>

<div id="fanChip" class="chip">OFF</div>

<div class="sub">Follows active Peltier modes</div>

</article>

<article class="panel metric">

<div class="panel-header">

<h2>Lights</h2>

</div>

<div id="lightsChip" class="chip">OFF</div>

<div id="lightsSub" class="sub">D7 left, D8 right</div>

</article>

<article class="panel log-panel">

<div class="panel-header">

<h2>Serial JSON Log</h2>

<span id="packetCount" class="chip">--</span>

</div>

<pre id="serialLog" class="empty-log">No serial data yet.</pre>

<div id="packetSub" class="sub">Receiver packet count</div>

</article>

</section>

<aside class="side-stack" aria-label="Dashboard controls and technology">

<section class="panel side-panel">

<div class="panel-header">

<h2>Buzzer Tone</h2>

<div class="value"><span id="buzzerValue">1200</span><span class="unit">Hz</span></div>

</div>

<div class="control-row">

<div>

<label for="buzzerRange">Frequency</label>

<input id="buzzerRange" type="range" min="200" max="5000" step="50" value="1200">

</div>

<div>

<label for="buzzerInput">Hz</label>

<input id="buzzerInput" type="number" min="200" max="5000" step="50" value="1200">

</div>

<button id="sendBuzzerButton" disabled>Send Tone</button>

</div>

<div id="buzzerSub" class="sub">Connect the bridge before sending config</div>

</section>

<section class="panel side-panel">

<div class="panel-header">

<h2>Technologies Used</h2>

</div>

<div class="stack-list">

<div class="stack-row">

<div class="stack-key">Hardware</div>

<div class="stack-value">3 Arduino Nano boards, nRF24L01 radios, DHT22 sensor, passive buzzer, directional LED outputs.</div>

</div>

<div class="stack-row">

<div class="stack-key">Arduino</div>

<div class="stack-value">C/C++ sketches using SPI, RF24, Servo, DHT, tone/noTone, packed command and telemetry structs.</div>

</div>

<div class="stack-row">

<div class="stack-key">Radio</div>

<div class="stack-value">nRF24L01 at channel 76, RF24_250KBPS, command pipe and dashboard telemetry pipe.</div>

</div>

<div class="stack-row">

<div class="stack-key">Browser</div>

<div class="stack-value">HTML5, CSS custom properties, vanilla JavaScript, Web Serial API, JSON Lines over USB Serial at 115200 baud.</div>

</div>

<div class="stack-row">

<div class="stack-key">Runtime</div>

<div class="stack-value">Local HTTP server on localhost. No ESP32, Bluetooth module, cloud database, npm package, or build step required.</div>

</div>

</div>

</section>

</aside>

</main>

</div>

<script>

const els = {

connectButton: document.getElementById('connectButton'),

disconnectButton: document.getElementById('disconnectButton'),

connectionDot: document.getElementById('connectionDot'),

connectionStatus: document.getElementById('connectionStatus'),

temperatureValue: document.getElementById('temperatureValue'),

temperatureSub: document.getElementById('temperatureSub'),

humidityValue: document.getElementById('humidityValue'),

humiditySub: document.getElementById('humiditySub'),

peltierMode: document.getElementById('peltierMode'),

peltierSub: document.getElementById('peltierSub'),

failsafeChip: document.getElementById('failsafeChip'),

signalSub: document.getElementById('signalSub'),

signalBar: document.getElementById('signalBar'),

motorValue: document.getElementById('motorValue'),

motorBar: document.getElementById('motorBar'),

motorSub: document.getElementById('motorSub'),

servoValue: document.getElementById('servoValue'),

steeringBar: document.getElementById('steeringBar'),

joystickSub: document.getElementById('joystickSub'),

fanChip: document.getElementById('fanChip'),

lightsChip: document.getElementById('lightsChip'),

lightsSub: document.getElementById('lightsSub'),

buzzerValue: document.getElementById('buzzerValue'),

buzzerRange: document.getElementById('buzzerRange'),

buzzerInput: document.getElementById('buzzerInput'),

sendBuzzerButton: document.getElementById('sendBuzzerButton'),

buzzerSub: document.getElementById('buzzerSub'),

packetCount: document.getElementById('packetCount'),

packetMini: document.getElementById('packetMini'),

packetSub: document.getElementById('packetSub'),

serialLog: document.getElementById('serialLog')

};

let port;

let reader;

let writer;

let keepReading = false;

let isConnecting = false;

let lineBuffer = '';

let hasSerialData = false;

const JOYSTICK_CENTER = 512;

const JOYSTICK_DEADZONE = 90;

const JOYSTICK_X_REVERSED = true;

const JOYSTICK_Y_REVERSED = true;

function setChip(el, label, state) {

el.textContent = label;

el.className = `chip ${state || ''}`.trim();

}

function setConnection(label, state) {

els.connectionStatus.textContent = label;

els.connectionDot.className = `dot ${state || ''}`.trim();

}

function serialErrorMessage(error) {

const message = error && error.message ? error.message : 'Serial connection failed';

if (message.includes('Failed to open serial port')) {

return 'Could not open COM port. Close Arduino Serial Monitor/IDE, other dashboard tabs, and any app using the Nano.';

}

if (message.includes('No port selected')) {

return 'No serial port selected';

}

return message;

}

async function cleanFailedSerialOpen() {

keepReading = false;

if (writer) {

writer.releaseLock();

writer = null;

}

if (port) {

try {

await port.close();

} catch {

// The port may not have opened far enough to close.

}

port = null;

}

els.connectButton.disabled = false;

els.disconnectButton.disabled = true;

els.sendBuzzerButton.disabled = true;

}

function setMeter(el, percent) {

const safePercent = Math.max(0, Math.min(100, Number(percent) || 0));

el.style.width = `${safePercent}%`;

}

function appendLog(line) {

const time = new Date().toLocaleTimeString();

if (!hasSerialData) {

els.serialLog.textContent = '';

els.serialLog.classList.remove('empty-log');

hasSerialData = true;

}

els.serialLog.textContent = `[${time}] ${line}\n` + els.serialLog.textContent;

const lines = els.serialLog.textContent.split('\n');

if (lines.length > 80) {

els.serialLog.textContent = lines.slice(0, 80).join('\n');

}

}

function displayNumber(value, digits = 0) {

if (typeof value !== 'number' || Number.isNaN(value)) {

return '--';

}

return value.toFixed(digits);

}

function correctedJoystickValue(value, reversed) {

if (typeof value !== 'number' || Number.isNaN(value)) {

return null;

}

const safeValue = Math.max(0, Math.min(1023, value));

return reversed ? 1023 - safeValue : safeValue;

}

function correctedJoystick(data) {

return {

x: correctedJoystickValue(data.x, JOYSTICK_X_REVERSED),

y: correctedJoystickValue(data.y, JOYSTICK_Y_REVERSED)

};

}

function directionFromAxis(value, negativeLabel, positiveLabel) {

if (value === null) {

return '--';

}

if (value < JOYSTICK_CENTER - JOYSTICK_DEADZONE) {

return negativeLabel;

}

if (value > JOYSTICK_CENTER + JOYSTICK_DEADZONE) {

return positiveLabel;

}

return 'CENTER';

}

function driveDirection(value) {

if (value === null) {

return '--';

}

if (value < JOYSTICK_CENTER - JOYSTICK_DEADZONE) {

return 'REVERSE';

}

if (value > JOYSTICK_CENTER + JOYSTICK_DEADZONE) {

return 'FORWARD';

}

return 'NEUTRAL';

}

function joystickLabel(data) {

const joystick = correctedJoystick(data);

const steering = directionFromAxis(joystick.x, 'LEFT', 'RIGHT');

const drive = driveDirection(joystick.y);

const xLabel = joystick.x === null ? '--' : joystick.x;

const yLabel = joystick.y === null ? '--' : joystick.y;

return `Corrected X: ${xLabel} - Y: ${yLabel} (${steering}, ${drive})`;

}

function lightModeFromData(data) {

if (typeof data.lightsMode === 'string') {

return data.lightsMode;

}

if (typeof data.lightsActive === 'number') {

switch (data.lightsActive) {

case 1:

return 'BOTH_BLINK';

case 2:

return 'RIGHT_BLINK';

case 3:

return 'LEFT_BLINK';

case 4:

return 'BOTH_ON';

default:

return 'OFF';

}

}

return data.lightsActive ? 'BOTH_ON' : 'OFF';

}

function lightModeLabel(mode) {

switch (mode) {

case 'BOTH_BLINK':

return 'BOTH BLINK';

case 'RIGHT_BLINK':

return 'RIGHT BLINK';

case 'LEFT_BLINK':

return 'LEFT BLINK';

case 'BOTH_ON':

return 'BOTH ON';

default:

return 'OFF';

}

}

function lightModeState(mode) {

if (mode === 'BOTH_ON') {

return 'ok';

}

if (mode === 'BOTH_BLINK' || mode === 'RIGHT_BLINK' || mode === 'LEFT_BLINK') {

return 'warn';

}

return '';

}

function lightModeSubtext(mode) {

switch (mode) {

case 'BOTH_BLINK':

return 'Left and right blinking';

case 'RIGHT_BLINK':

return 'Right directional blinking';

case 'LEFT_BLINK':

return 'Left directional blinking';

case 'BOTH_ON':

return 'Left and right solid on';

default:

return 'D7 left, D8 right';

}

}

function applyLightMode(data) {

const mode = lightModeFromData(data);

setChip(els.lightsChip, lightModeLabel(mode), lightModeState(mode));

els.lightsSub.textContent = lightModeSubtext(mode);

}

function applyTelemetry(data) {

if (data.sensorValid && typeof data.temperatureC === 'number') {

els.temperatureValue.textContent = displayNumber(data.temperatureC, 1);

els.temperatureSub.textContent = 'Live cabin sensor';

} else {

els.temperatureValue.textContent = '--';

els.temperatureSub.textContent = 'Sensor not ready';

}

if (data.sensorValid && typeof data.humidity === 'number') {

els.humidityValue.textContent = displayNumber(data.humidity, 1);

els.humiditySub.textContent = 'Live cabin sensor';

} else {

els.humidityValue.textContent = '--';

els.humiditySub.textContent = 'Sensor not ready';

}

const mode = data.peltierMode || 'OFF';

const modeState = mode === 'HEAT' ? 'warn' : mode === 'COLD' ? 'ok' : '';

setChip(els.peltierMode, mode, modeState);

els.peltierSub.textContent = `Fan command: ${data.fanActive ? 'ON' : 'OFF'}`;

setChip(els.failsafeChip, data.failsafe ? 'ACTIVE' : 'CLEAR', data.failsafe ? 'danger' : 'ok');

els.signalSub.textContent = `Signal age: ${data.signalAgeMs ?? '--'} ms`;

setMeter(els.signalBar, Math.min(100, ((data.signalAgeMs || 0) / 1000) * 100));

els.motorValue.textContent = displayNumber(data.motorPWM, 0);

setMeter(els.motorBar, ((data.motorPWM || 0) / 255) * 100);

const corrected = correctedJoystick(data);

const driveLabel = driveDirection(corrected.y);

els.motorSub.textContent = data.motorPWM > 0 ? `${driveLabel} command active` : `${driveLabel} / stopped`;

els.servoValue.textContent = displayNumber(data.servoPulse, 0);

setMeter(els.steeringBar, (((data.servoPulse || 1500) - 1000) / 1000) * 100);

els.joystickSub.textContent = joystickLabel(data);

setChip(els.fanChip, data.fanActive ? 'ON' : 'OFF', data.fanActive ? 'ok' : '');

applyLightMode(data);

if (typeof data.buzzerToneHz === 'number') {

els.buzzerValue.textContent = data.buzzerToneHz;

els.buzzerRange.value = data.buzzerToneHz;

els.buzzerInput.value = data.buzzerToneHz;

}

const packetLabel = data.packetCount ?? '--';

els.packetCount.textContent = packetLabel;

els.packetMini.textContent = `PACKETS ${packetLabel}`;

els.packetSub.textContent = data.buzzerActive ? 'Buzzer button is pressed' : 'Buzzer button is off';

}

function applyCommand(data) {

const mode = data.peltierMode || 'OFF';

setChip(els.peltierMode, mode, mode === 'HEAT' ? 'warn' : mode === 'COLD' ? 'ok' : '');

setChip(els.fanChip, data.fanActive ? 'ON' : 'OFF', data.fanActive ? 'ok' : '');

applyLightMode(data);

els.joystickSub.textContent = joystickLabel(data);

}

function handleLine(line) {

const trimmed = line.trim();

if (!trimmed) {

return;

}

appendLog(trimmed);

let data;

try {

data = JSON.parse(trimmed);

} catch {

return;

}

if (data.type === 'telemetry') {

applyTelemetry(data);

} else if (data.type === 'command') {

applyCommand(data);

} else if (data.type === 'config') {

els.buzzerSub.textContent = data.ok ? `Tone sent: ${data.buzzerToneHz} Hz` : 'Receiver did not acknowledge tone';

} else if (data.type === 'bridge') {

setConnection(`Bridge ${data.status}`, data.status === 'ready' || data.status === 'ok' ? 'online' : 'warn');

}

}

async function connectSerial() {

if (isConnecting || port) {

return;

}

if (!('serial' in navigator)) {

setConnection('Use Chrome or Edge with Web Serial support', 'danger');

return;

}

isConnecting = true;

els.connectButton.disabled = true;

setConnection('Selecting USB bridge...', 'warn');

try {

port = await navigator.serial.requestPort();

setConnection('Opening serial port...', 'warn');

await port.open({ baudRate: 115200 });

writer = port.writable.getWriter();

keepReading = true;

isConnecting = false;

els.disconnectButton.disabled = false;

els.sendBuzzerButton.disabled = false;

setConnection('Connected at 115200 baud', 'online');

els.buzzerSub.textContent = 'Ready to send buzzer tone';

} catch (error) {

isConnecting = false;

await cleanFailedSerialOpen();

setConnection(serialErrorMessage(error), 'danger');

return;

}

const decoder = new TextDecoder();

try {

while (port && port.readable && keepReading) {

reader = port.readable.getReader();

try {

while (true) {

const { value, done } = await reader.read();

if (done) {

break;

}

lineBuffer += decoder.decode(value, { stream: true });

const lines = lineBuffer.split('\n');

lineBuffer = lines.pop();

for (const line of lines) {

handleLine(line);

}

}

} finally {

reader.releaseLock();

reader = null;

}

}

} catch (error) {

if (!keepReading) {

return;

}

await cleanFailedSerialOpen();

setConnection(serialErrorMessage(error), 'danger');

}

}

async function disconnectSerial() {

keepReading = false;

if (reader) {

await reader.cancel();

}

if (writer) {

writer.releaseLock();

writer = null;

}

if (port) {

await port.close();

port = null;

}

els.connectButton.disabled = false;

els.disconnectButton.disabled = true;

els.sendBuzzerButton.disabled = true;

setConnection('Disconnected', '');

}

async function sendLine(line) {

if (!writer) {

return;

}

const encoder = new TextEncoder();

await writer.write(encoder.encode(line));

}

function syncBuzzerInputs(value) {

const next = Math.max(200, Math.min(5000, Number(value) || 1200));

els.buzzerRange.value = next;

els.buzzerInput.value = next;

els.buzzerValue.textContent = next;

}

els.connectButton.addEventListener('click', () => {

connectSerial();

});

els.disconnectButton.addEventListener('click', () => {

disconnectSerial().catch((error) => {

setConnection(error.message, 'danger');

});

});

els.buzzerRange.addEventListener('input', (event) => syncBuzzerInputs(event.target.value));

els.buzzerInput.addEventListener('input', (event) => syncBuzzerInputs(event.target.value));

els.sendBuzzerButton.addEventListener('click', () => {

const tone = Number(els.buzzerInput.value) || 1200;

syncBuzzerInputs(tone);

sendLine(`BUZZER ${els.buzzerInput.value}\n`).catch((error) => {

els.buzzerSub.textContent = error.message;

});

});

</script>

</body>

</html>