| Safe Operating Procedures Bench Grinder BENCH SHEARS |  |
| --- | --- |
| DO NOT use this machine unless you have been fully trained on its safe use and operation. |  |
| FACE SHIELD must be worn at all times in work areas. | Long and loose hair must be contained or restrained. |
| Appropriate protective footwear with substantial uppers must be worn. | Leather apron and close-fitting clothing must be worn. |
| Rings and jewellery must not be worn. | No gloves are worn. |

Materials: Steels, irons, ferrous metals. NO ALUMINIUM OR BRASS / OTHER SOFT METAL.

### PRE-OPERATIONAL SAFETY CHECKS

- Locate and ensure you are familiar with all machine operations and controls.

- Ensure your metal is suitable for this tool! Do not attempt to grind metal that is difficult to hold, awkwardly shaped, or too soft for this machine

- Ensure all guards are fitted, secure and functional. Do not operate if guards are missing or faulty.

- Ensure grinding wheels have no cracks or other deformities, they may explode if they do.

- Check workspaces and walkways to ensure no slip/trip hazards are present

- Be aware of other people in the area.

- Ensure the area is clear before using equipment.

- Check for fire hazards as sparks will be generated.

- Have a method to cool the metals.

#### OPERATIONAL SAFETY CHECKS

- Always place workpiece against table stop, do not allow piece to float.

- Ensure guard will not fall during work.

- Do not angle workpiece down towards the ground.

- Ensure gap between table and grinding wheel is small enough to not allow workpiece through.

- Wear all PPE (safety glasses, face shield, leather apron).

- Ensure the wheel used is proper for the task (do not use a polishing wheel for material removal).

#### HOUSEKEEPING

- Check the condition of all wheels.

- Return all accessories to storage racks.

- Leave the work area in a safe, clean, and tidy state

## POTENTIAL HAZARDS

Sparks						 Propelled materials					  Manual handling

Crush and pinch points		 Abrasives