// Reference: https://www.rcpano.net/2021/05/25/how-to-make-remote-control-for-model-car-boat-diy-3-channel-rc/#google_vignette

// Libraries (make sure they are all properly installed)

#include <SPI.h>

#include <nRF24L01.h>

#include <RF24.h>

#include <Servo.h>

#define motor 5

// D5 to motor MOSFET gate

#define servo 3

// D3 to servo

#define peltierMode 2

// D2 to relay that controls heat/cool mode

#define peltierPower 6

// D6 to Peltier MOSFET gate

#define ledCold 7

// D7 cool mode LED

#define ledHot 8

// D8 heat mode LED

#define buzzerPin A0

// A0 for buzzer

int ch_width_1 = 0;

int ch_width_2 = 0;

Servo ch1;

struct Signal {

byte steering;

byte throttle;

byte button; //new

};

Signal data;

const uint64_t pipeIn = 0xE9E8F0F0E2;

// Team 1: 0xE9E8F0F0E1

// Team 2: 0xE9E8F0F0E2

// Team 3: 0xE9E8F0F0E3

// Team 4: 0xE9E8F0F0E4

RF24 radio(9, 10); // CE,CSN pins

int mode = 0; // 0 = off, 1 = cool, 2 = heat

bool lastButtonState = false;

void ResetData()

{

// Define the initial value of each data input. The middle position for Potentiometers.

data.steering = 127; // Center

data.throttle = 0; // Motor Stop

data.button = 0; // Joystick Button

}

void setup()

{

//Set the pins for each PWM signal

ch1.attach(servo);

pinMode(motor, OUTPUT);

pinMode(peltierMode, OUTPUT);

pinMode(peltierPower, OUTPUT);

pinMode(ledCold, OUTPUT);

pinMode(ledHot, OUTPUT);

pinMode(buzzerPin, OUTPUT); // Configure A0 as output for buzzer

digitalWrite(peltierMode, LOW);

analogWrite(peltierPower, 0);

digitalWrite(ledCold, LOW);

digitalWrite(ledHot, LOW);

noTone(buzzerPin); // Ensure passive buzzer is off initially

//Configure the NRF24 module

ResetData();

radio.begin();

radio.openReadingPipe(1,pipeIn);

radio.setAutoAck(false);

radio.setDataRate(RF24_250KBPS);

radio.setPALevel(RF24_PA_MAX);

radio.startListening(); //start the radio comunication for receiver

}

unsigned long lastRecvTime = 0;

void recvData()

{

while ( radio.available() ) {

radio.read(&data, sizeof(Signal));

lastRecvTime = millis();   // receive the data

}

}

void updatePeltierMode() {

bool buttonPressed = data.button == 1;

if (buttonPressed && !lastButtonState) {

mode = (mode + 1) % 3;

switch (mode) {

case 0: // Off

analogWrite(peltierPower, 0); // Peltier off

digitalWrite(peltierMode, LOW); // relay off

digitalWrite(ledCold, LOW);

digitalWrite(ledHot, LOW);

break;

case 1: // Cool Mode

analogWrite(peltierPower, 127); // Peltier on 50%

digitalWrite(peltierMode, LOW); // Cool mode selected

digitalWrite(ledCold, HIGH);

digitalWrite(ledHot, LOW);

break;

case 2: // Heat Mode

analogWrite(peltierPower, 127); // Peltier on 50%

digitalWrite(peltierMode, HIGH); // Heat mode selected

digitalWrite(ledCold, LOW);

digitalWrite(ledHot, HIGH);

break;

}

}

lastButtonState = buttonPressed;

}

void updateBuzzer() {

static unsigned long lastToggle = 0;

const unsigned long beepInterval = 200; // Beep every 200ms for noticeability

if (data.steering < 117 || data.steering > 137) {

if (millis() - lastToggle >= beepInterval) {

if (digitalRead(buzzerPin) == LOW) { // Check if tone is off

tone(buzzerPin, 2500); // 2.5kHz tone for passive buzzer

} else {

noTone(buzzerPin); // Stop tone

}

lastToggle = millis();

}

} else {

noTone(buzzerPin); // Ensure buzzer is off when centered

}

}

void loop()

{

recvData();

unsigned long now = millis();

if ( now - lastRecvTime > 1000 ) {

ResetData(); // Signal lost. Reset data.

}

ch_width_1 = map(data.steering, 15, 240, 500, 2500);     // pin D3 (PWM signal)

//ch_width_1 = map(data.steering, 15, 240, 1000, 2000);     // for some models of servo

ch_width_2 = map(data.throttle, 140, 255, 0, 255); // Only brighten when throttle > 140

ch_width_2 = constrain(ch_width_2, 0, 255);        // Ensure valid PWM range

//Write the PWM signal.

ch1.writeMicroseconds(ch_width_1);

analogWrite(motor, ch_width_2);

updatePeltierMode();

updateBuzzer(); // Update buzzer state based on steering

}