| Safe Operating Procedures 12in Wood Lathe BENCH SHEARS |  |
| --- | --- |
| DO NOT use this machine unless you have been fully trained on its safe use and operation. |  |
| Safety glasses and dust masks must be worn at all times. | Long and loose hair must be contained or restrained. |
| Appropriate protective footwear with substantial uppers must be worn. | Close fitting and protective clothing must be worn. |
| Rings and jewellery must not be worn. | No gloves must be worn. |
|  | Compatible Materials: Wood |

### PRE-OPERATIONAL SAFETY CHECKS

- Locate and ensure you are familiar with all machine operations and controls.

- Ensure all guards and rests are fitted, secure and functional. Do not operate if guards are missing or faulty.

#### OPERATIONAL SAFETY CHECKS

- Remove tailstock and use to mark centre of turning piece.

- Replace and secure head and tail stock.

- Ensure workpiece will not hit any objects during turning.

- Confirm height of tool rest based on cutting tool and operation.

- Ensure head and tail stock are aligned.

- Do not grab or place hands near workpiece while machine is powered.

- Ensure sleeves are not near workpiece and all gloves / jewellery have been removed.

- Take care to prevent catching of tools and maintain contact with the tool rest at all times.

#### HOUSEKEEPING

- Shut down machine and allow it to spin to a stop. Remove power plug from wall.

- Clean up all debris.

- Return all guards / rests to the initial position.

- Lock power switch.

## POTENTIAL HAZARDS

Crush / rotary injuries		 Severe cuts / amputations	  Inhalants